14	Suspended cubicle system cronus

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:
The suspension profile is held above the wall connection

and/or the floor-to-ceiling support column.

The number of floor-to-ceiling support columns depends

on the cubicle system type and width. All pilasters (front

panels)  hang from suspension profile with concealed

screw fixings. The aluminium suspension profile is fixed

with wall connection parts to prefabricated  brick work

with heavy duty dowels.

In the case of plasterboard walls, reinforcement inlays

(to be provided by the customer) are essential.

The floor-to-ceiling support column is fixed to the floor.

Height:
2000 mm including 150 mm under panel clearance

to the bottom edge of the suspension profile.

Other heights are available.

Suspension profile:

Aluminium rectangular profiles in the dimensions

50x150x5 mm without edge radius.

For bigger ranges, aluminium rectangular profiles

in the dimensions  50x200x5 mm without edge radius.

Corner profiles:

Aluminium square tube in the dimensions 50x50 mm

from floor to the top edge of the suspension profile.

Floor-to-ceiling support columns:

Aluminium square tube inside the cubicle with the

dimensions 50x50 mm from floor to the top edge of

the suspension profile.

Colours:
Suspension profile, corner profiles and floor-to-ceiling

support column in chrome finish and

powder-coated according to KEMMLIT colour chart.

Surcharge for suspended design.

All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included./E$$GRUEnd

$(kurz9Suspended cubicle system cronus $)	$(gb.............$)

============