$(h1ii1$)	$(kurz9Cubicle system cronus NR$)

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:

Cubicle system made of 13 mm HPL solid grade laminate

in a static connection to the

aluminium profiles.

TÜV-approved (GS-seal of approval).

Supporting legs integrated into the front.

Also available:

Support-optimised and front hanging solution

with supporting legs integrated into the partition.

Suspended  cubicles with suspension system,

without supporting legs as an easy-to-clean option.

Floor-to-ceiling cubicles with no under panel clearance.

Options:
CR 2.1: Profiles anodised.

CR 2.2: Profiles powder-coated.

Height:
2000 mm, including 150 mm under panel clearance.

2135 mm, including 150 mm under panel clearance.

Other heights are also available.

Partitions and pilasters (front panels):
13 mm thick HPL solid grade laminate according to

DIN EN 438 (DIN 16926). Water-, decay- and

damage-resistant and especially low maintenance.

All visible edges are rounded to reduce the risk of injuries.

Profiles:
All profiles are made of aluminium. Warp-resistant

rectangular 3-chamber-stabilisator head rail (54/43 mm

wall connection cover plates above the door fronts.

The head rail is attached with M12x50 screw

fittings in an interlock connection to the smooth-surfaced

hinge profile and door stop profile.

3-chamber-door stop profiles (50/37 mm) are warp-resistant

and force-fit connected to the head rail.

The underside of the hinge and door stop profiles are

concealed by an aluminium cover plate,

which reduces the risk of injury. Wall connection by means of

U-channel profiles (30/24 mm).

Supporting legs:

Aluminium supporting legs (20 mm in diameter) with

stainless steel thread and stainless steel base plate.

Adjustable in height. Force-fit connection by means of

M12 screw fittings to the hinge and door-stop profiles.

Durable corrosion-resistant satin anodised aluminium

cover sleeves can be lifted for cleaning purposes.

Alternatively durable cover sleeve made of

colour-coordinated nylon. Cover sleeve can easily be

lifted for cleaning purposes.

Doors:
13 mm thick solid grade laminate (HPL) panels

sit flush with the front. All edges bevelled and the

corners are rounded with a 2 mm radius.

The doorstop profiles are fitted with a rubber seal

to reduce closing noise.
 
As an option, the doors can be provided with finger

protection on the hinge side, engravings, cut-outs,

aluminium doors or glass doors (toughened safety

glass ESG).

Furniture/hardware:

3 robust 3-roller-edged hinges made of aluminium

(20 mm in diameter). One hinge provided with

integrated  spring, for either self-closing or

self-opening doors. Hinges attached to the surface of

the door to prevent splitting. Concealed stainless steel

TORX screws are used to fix the hinges.

Plastic-free single latch lock made of zinc die-cast

with solid, non-flammable aluminium latch.

Lock cover plate made of solid aluminium, with latch

turn on the inside and red/white indicator with

emergency release mechanism on the outside.

Door knobs with grip made of  satin anodised

aluminium.

As an option single-handed turning knob made of satin

anodised aluminium.

As a further option, door knobs and a latch turn

made of colour-coordinated, impact- and

shatter-resistant nylon.

Colours:
Colours according to the KEMMLIT colour chart.

Profiles, hardware, supporting legs and accessories

available in the same colours according to the KEMMLIT

colour chart.

All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included.$)

1.1		                                                    $)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	   $(gb.............

Surcharges for further options:

Legs fitted on partitions

1.11 		                                                    $)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	   $(gb.............

Suspended construction

1.12 			$)		$	(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

Cut-out in HPL with inlayed glass / Plexiglas

1.13 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

Engravings in HPL

1.14 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

Overhead panels made of HPL

1.15 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

Overhead light elements made of glass / Plexiglas

1.16 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

Swinging door made of HPL (saloon door)

1.17 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

Door made of toughened safety glass ESG

1.18 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

Door made of aluminium

1.19 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

/E$$NNN

$(h1ii

$(langAccessories for  WC-cubicles:

KEMMLIT- door stopper hook
made of nylon 301$)/E

$$NNN

$(h1ii1.20$)			$(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)		            $(gb.............

$)

$(langKEMMLIT-wall hook

made of nylon 302$)/E

$$NNN

$(h1ii1.30$)			                             $(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)	                         $(gb.............

$)

$(langKEMMLIT-toilet roll holder

made of nylon 308$)/E

$$NNN

$(h1ii1.40$)			                             $(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)	                         $(gb.............

KEMMLIT-replacement toilet roll holder

made of nylon 309

$(h1ii1.50$)			                             $(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)	$(gb.............$)

$(langKEMMLIT- square-shaped toilet brush

made of nylon 310

$)/E$$NNN

$(h1ii1.60$)			                             $(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)	                         $(gb.............

Accessories also available in stainless steel $)

$(lang$)

$$NNN

$(langAccessories for changing cubicles:

KEMMLIT-cubicle bench seating for changing

cubicles made of 13 mm thick HPL panels.

$$NNN

$(h1ii1.70$)			                             $(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)		            $(gb.............

$)

$(langKEMMLIT-central under bench seat locking

system or overhead locking for simultaneously

closing of both cubicle doors, with maintenance-

free rack construction, with emergency release

mechanism and clearly visible vacant / occupied

indicator (red/white).$)

$$NNN

$(h1ii1.80$)			                             $(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)		            $(gb.............

$)

$(langKEMMLIT- door stopper hook

for changing cubicles made of nylon 301$)/E

$$NNN

$(h1ii1.90$)			                             $(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)		            $(gb.............

$)

$(langKEMMLIT crystal mirror for changing cubicles

with sealed edges,  8 mm thick, edges rounded,

size 300 x 400 mm. Mirror glued on HPL is

not allowed. Theft-proof screw connection.$)/E

$$NNN

$(h1ii1.100$)			                             $(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)		            $(gb.............

$)

$(langKEMMLIT-shelf for changing cubicles
made of nylon 312, size 300 x 50 mm.

Theft-proof screw connection.$)

/E

$$NNN

$(h1ii1.110$)			                             $(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)		            $(gb.............

$)

$(langKEMMLIT-anti-climb protection for changing

cubicles. Theft-proof screw connection.$)

$(h1ii1.120$)			                             $(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)	$(gb.............$)

Cubicle system cronus NR /E$$GRUEnd

/E