5$(h1ii0$)	$(kurz9Cubicle system Cronus Glass$)

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:

Cubicle system made of toughened safety

glass (ESG) in a static connection

with the aluminium profiles.

TÜV-approved (GS-seal of approval).

Supporting legs in the front.

Support optimised and front-hanging systems

with supporting legs set back in the partitions.

Construction with overhead panels or glass elements.

Option:
CR 6.1: Profiles anodised.

Height:
2000 mm, including 150 mm under panel clearance.

2135 mm, including. 150 mm under panel clearance.

Other heights are available.

Partitions and pilasters (front panels):

Side panels and pilasters  (front panels) consist of

12 mm thick, toughened safety glass (ESG).  Partitions consist

of 8 mm thick, toughened safety glass (ESG).

Profiles:
All profiles made of aluminium. Warp-resistant

rectangular 3-chamber-stabilisator head rail (54/43 mm)

with wall connection cover plates above the door fronts.

Stabilisator head rail is connected by M12x50 screw

fittings in a static connection to the flush

hinge and door stop profiles. 3-chamber door stop profiles

(50/37 mm) are warp-resistant  and are connected

by a force-fit connection to the stabilisator head rail.

The under side of the hinge and door stop profiles is concealed

by an aluminium cover plate, which reduces the risk of injury.

Pilasters (front panels and side panels are connected to the wall

by means of U-channel profiles (30/24 mm). Partitions are fixed

with aluminium brackets.

Supporting legs:
Aluminium supporting legs (20 mm in diameter) with

stainless steel thread and stainless steel plate.

Adjustable in height. Force–fit connection by means of

M12 screw fittings to the hinge and door-stop profiles.

Durable corrosion-free aluminium cover sleeves

satin anodised can be easily lifted for cleaning purposes.

Doors:
The doors are made of toughened safety glass (ESG) 8 mm.

Door stop profiles integrated with a sound absorption rubber

stop to reduce closing noise.

Furniture/hardware:
Two solid 3-roller edge hinges made of aluminium, (20 mm in

diameter). One hinge fitted with integrated spring for self-closing

or self-opening of the door. Hinges are fixed to the surface with

special screws for glass.

Plastic-free single latch lock made of zinc die-cast

with solid, non-flammable aluminium latch.

Lock cover plate made of solid aluminium, with latch turn

on the inside and red/white indicator with emergency

release mechanism on the outside.  Door knobs with

recessed grip made of satin anodised aluminium.

As an option single handed turning knob made of satin anodised

aluminium.

As an option with drillings for grip hole (55 mm in diameter)

without lock.

Colours:
Choice of five colours according to the KEMMLIT colour range.

Profiles, hardware, glass screws and supporting legs in satin

anodised aluminium.

Cubicle system  consisting of:

Pilaster (front panel):		            ............ mm 	.......... No.

Doors with hardware:			                          .......... No.

Partitions:	                                      		............. mm 	.......... No.

Number of sets:$(einhQuantit			........... $(menge$(einh No. $)	$(d276$)$(ep

All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included.

$)	$(d276$)$(ep............. 	            .............$)

Further options at a surcharge:

Front-hanging option

50.11 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

Overhead panels made of HPL or glass

50.12 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

Light element made of  glass or plexiglas

50.13 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

/E$$NNN

$(h1ii

$(langAcces		           Accessories for WC cubicles:
$(h1ii$)

KEMMLIT door stopper hooks made of stainless steel 501

50.20 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

KEMMLIT-Wall hooks small made of stainless steel 503

50.30 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

KEMMLIT-toilet roll holder made of stainless steel 508

50.40 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

KEMMLIT replacement toilet roll holder made of stainless steel

50.50 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

KEMMLIT-Toilet brush set made of stainless steel

511

50.60 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

$(lang$)$$NNN

$)$)

/E$$GRUEnd

----------------------

Cubicle system$(kurz9Cubicle  cronus glass$)	$(gb.............$)

============

/E