71	Cubicle System PRIMO KN

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:

42 mm HPL- composite elements,

solidly connected profiles and

vandalism-proof hardware.

TÜV-approved (GS-seal of approval).30 mm starke Schicht

Options:
PRIMO KN 2.1: Anodised aluminium profiles 
PRIMO KN 2.2: Powder-coated profiles

Height:
2000 mm including 150 mm under panel clearance

Optional: 2135mm including 150 mm under panel clearance

Other heights available upon request.

Partition walls and front panels (pilasters):
42 mm HPL- composite elements,

both sides made of 3 mm HPL- surface (core made of polystyrol),

with an internal aluminium frame for maximum torsion resistance and longevity.

Profiles:
All profiles are rounded for safety reasons (radius 2 mm).

The torsion resistant, flush stabilisor (42/42 mm) above the door is

made of high-quality aluminium.

- Wall connections via U-profiles to allow for construction tolerances.

Supporting legs:
Legs made of anodised aluminium (Ø 20 mm), with a

stainless steel foot plate, infinitely adjustable, firmly screwed onto the cubicle

elements using a M12 thread.

The solid, corrosion-free aluminium covering rosette can be easily lifted

for cleaning purposes.

Optional: Legs made entirely of stainless steel (Ø 20 mm) with a stainless

steel foot plate, infinitely adjustable.

The solid, covering rosette made of stainless steel can be easily lifted

for cleaning purposes.

Doors:
42 mm HPL-composite elements, same construction as wall elements.

Rebated doors with flush connections to the front walls.

Hinges:
2 robust, 3-roller edge spring hinges made of aluminium

(20mm in diameter, length 145 mm) with a

maintenance free stainless steel center.

Hinges are non-detachable , fastened with six screws

and come with a stainless steel pin for self-closing doors.

Optional: 2 robust, 3-roller edge spring hinges made of stainless steel

(20mm in diameter, length 145 mm) with a maintenance free stainless steel center,

hinges are non-detachable , fastened with six screws

and come with a stainless steel pin for self-closing doors

Locks and door knobs / handles:

Two alternatives are available:

- Double-latch lock with metal latch and

metal whisper trap for a silently closing door.

U-shaped lever handle and turning lock made

of nylon with square emergency lock release,

locking status visible inside and outside

Optional: A U-shaped lever handle made of anodized aluminium or stainless

steel or a L-shaped handle made of anodized aluminium or stainless steel

with square emergency lock release,

locking status visible inside and outside

- Single-latch lock with round door knobs made of nylon or anodised aluminium or

stainless steel. Lock with indicator disc red/white  and with square emergency lock release.

Colours:
Colours of partitions and pilasters/front panels

according to the KEMMLIT colour chart

(at least  20 colours).

Profiles, hinges, hardware, supporting legs and accessories

according to the KEMMLIT colour chart

(at least 8 colours).

All prices ex works Dusslingen. Taxes and duties

unpaid, materials only. Installation and transportation

are not included.

15.10 						0,000 no.	.............	.............

Cubicle Accessories:

KEMMLIT- door stopper hook

15.20           made of nylon   301	0,000  units		……......	.............

KEMMLIT - wall hook

15.30           made of nylon   302$)	                            0,000 units$)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT- toilet paper roll holder $)/E

$$NNN

$(h1ii15.40           made of nylon   308$)	                            $(menge0,000$) $(einhunits$)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT- spare roll holder $)/E

$$NNN

$(h1ii15.50           made of nylon   309$)	                            $(menge0,000$) $(einhunits$)	$(d276$)$(ep.............$)	$(gb.............$)

$(langKEMMLIT- toilet brush set 
 $)/E$$NNN

$(h1ii15.60 	          made of nylon  310$)	                            $(menge0,000$) $(einhunits$)	$(d276$)$(ep.............$)	$(gb.............

- All accessories are also available in stainless steel