62$)	Cubicle system$(kurz9Cubicle  classiccell stainless steel$)

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:

Flush-surfaced stainless steel composite elements,

42 mm thick, with a static connection

and vandal-resistant hardware.

TÜV-approved (GS seal of approval).

Options:
classiccell stainless steel 4.1: anodised aluminium profiles 
classiccell stainless steel 4.4: stainless steel profiles

Height:
2000 mm, including 150 mm under panel clearance.
2135 mm, including 150 mm under panel clearance.

Other heights are also available.

With 80 mm floor and ceiling clearance, for maximum privacy

combined with highest cleaning friendliness.

Floor-to-ceiling construction with room-height doors

for a maximum privacy also available.

Partition panels and pilasters:

Stainless steel sandwich elements, 42 mm thick,

- DIN EN 10 027-2. The stainless steel sheets are canted

- on all four sides. The inside of the elements is filled with a

- warp-resistant inlay. Screw-proof inlays in the composite

- elements guarantee the long-lasting  and solid fixing of

accessories. Surface or structure according to the KEMMLIT

colour chart (stainless steel)

As an alternative with coloured finish according to the

KEMMLIT colour chart (stainless steel).

Profiles: 
The smooth, square shaped aluminium profile (head rail) 54/43

is situated visibly directly above the door front.

- As an alternative square satin anodised aluminium profile 40/30

- As an alternative square stainless steel profile 40/30

- As an alternative concealed head rail integrated in the

- door front.

Wall connections with surrounding U-profiles made of

satin anodised aluminium.

- As an alternative shadow gap wall connection with U-profiles.

Supporting legs:

Anodised aluminium supporting legs (Ø 20 mm), with

stainless steel core and foot plate, adjustable in height.

Force fit connection with the partition by means of a M12

screw connection. The corrosion resistant and durable

aluminium cover sleeve can be easily lifted for cleaning

purposes.

- As an alternative made completely of stainless steel (Ø 20 mm).

- Force fit connection with the partition by means of a M12

- screw connection. The corrosion resistant and durable

- stainless steel cover sleeve can be easily lifted for cleaning

- purposes.

Doors:
Stainless steel composite elements, 42 mm thick, DIN EN 10 027-2

(same construction as the partition panels and pilasters), rebated.

- Hinges:

2 maintenance-free, impossible to unhinge,

3-roller edge hinges made of satin anodised aluminium,

screwed six times, (Ø 20 mm) with hinge roller extended

to 145 mm. One hinge is equipped with an integrated spring

for either self-closing or self-opening of the doors.

As an alternative made of gloss polished aluminium.

As an alternative 2 maintenance-free, impossible to unhinge,

3-roller edge hinges made of brushed stainless steel

(Ø 20 mm), with hinge roller extended to 145 mm.

One hinge is equipped with an integrated spring

for either self-closing or self-opening of the doors.

- Furniture/ hardware:

Double latch lock WC. Quality lever handle tested according

to EN 1906 class 3 made of satin anodised aluminium.

Locking device made of satin anodised aluminium with

red/white indicator and metal emergency release bolt.

As an alternative made of stainless steel.

As an alternative made of nylon (colours according to the KEMMLIT colour chart).

Single latch lock WC. Door knobs made of satin anodised aluminium.

Locking device made of satin anodised aluminium with

red/white indicator and metal emergency release bolt.

As an alternative made of stainless steel.

As an alternative made of nylon (colours according to the KEMMLIT colour chart).

As an option: single latch lock with grab bar made of solid

aluminium available in 4 versions with LED-indicator and

erosion resistant metal bolt as emergency release mechanism.

Colours:
Panels and door colours according to the cell colour chart.

Environmental friendliness:
Only environmentally friendly materials without PVC or CFCs

must be used.

Prices ex works Dusslingen, excluding taxes and duties,

transportation and installation not included.$)

$(h1ii

Accessories for WC cubicles:$(lang:

KEMMLIT-door stopper hooks made of  stainless steel 5501$)/E

$$NNN

$(h1ii62.20$)			$(menge0,000$) No.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-wall hooks made of stainless steel 5502$)/E

$$NNN

$(h1ii62.30$)			                             $(menge0,000$) No.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-paper roll holders made of stainless steel 5508$)/E

$$NNN

$(h1ii62.40$)			                             $(menge0,000$) No.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-replacement toilet roll holders made of stainless steel 5509$)/E

$$NNN

$(h1ii62.50$)			                             $(menge0,000$) No.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............$)

$(langKEMMLIT-WC brush set made of stainless steel 5510
 $)/E$$NNN

$(h1ii62.60$)			                             $(menge0,000$) No.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

Accessories are also available in aluminium and nylon.

/E$$GRUEnd

----------------------

$(kurz9 Cubicle system classiccell stainless steel $)	$(gb.............

$)

============/E