$(h1ii58$)67	Cubicle system hardcell aluminium $(kurz9 $)

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:

Asymmetric folded aluminium sandwich element, 42 mm

thick, corrosion-resistant with  3-component varnish-coating

(Crystal Coat), with a static connection  and

vandal-resistant hardware. TÜV-approved (GS seal

of approval).

Options:
hardcell Aluminium 3.1: anodised aluminium profiles

hardcell Aluminium 3.4: stainless steel profiles

Height:
2000 mm, including 150 mm under panel clearance.
2135 mm, including 150 mm under panel clearance.

Other heights are also available.

Walls:
42 mm thick, corrosion-resistant aluminium sandwich element.

Environmentally friendly 3-component varnish-coating with a
non-sensitive structure (Crystal Coat), scratch- and

UV-resistant according to DIN 53384 A. The aluminium

alloy is seawater-resistant according to DIN 1725.

- The aluminium sheets are canted on all 4 sides.

The inside of the elements is filled with a warp-resistant inlay.

Additional screw-proof inlays in the sandwich elements guarantee

the long-lasting  and solid fixing of accessories.

- Profiles: 
The horizontal aluminium profile (frieze) does not run

- directly above the door front but rather is set back by 220mm

- Alternatively profile (frieze) available in stainless steel.

The wall connections are accomplished by shadow gaps.

Supporting legs:
The supporting legs are set back 220 mm.

Made entirely of stainless steel with a diameter of 20 mm.

Adjustable in height. Force-fit connection to the partitions

with M12 screw fitting. The durable stainless steel cover

sleeve can easily be lifted for cleaning purposes.

Also available in corrosion-resistant aluminium,

satin anodised with stainless steel core and

plate. Durable cover sleeve made

of anodised aluminium. Cover sleeve can easily be lifted

for cleaning purposes.

Doors:
Aluminium sandwich elements, 42 mm thick,

corrosion-resistant, rebated (same construction as

wall elements).

- Hinges:

2 robust, 2–roller-rising-butt-edge hinges made

of aluminium (20 mm in diameter), for automatic

closing of the door, satin anodised.

Alternatively 2 robust 2-roller edge hinges- (20mm

in diameter), made of aluminium satin anodised.

- Alternatively decorative 2-roller edge hinges made of

- stainless steel (12 mm in diameter).

- Furniture/hardware:

Single latch lock WC, door knobs, red/white

indicator bolt with emergency release mechanism

made of satin anodised aluminium, in stainless

steel or impact- and shatter-resistant nylon.

Double latch lock WC, safety lever handle (23 mm in diameter).

Red/white indicator bolt with emergency release mechanism

made of natural anodised aluminium, stainless steel

or impact and shatter-resistant and colour-coordinated

nylon.

Colours:
Colours of doors and walls according to the cell colour chart.

- $)

Environmental friendliness:
Only environmentally friendly materials without PVC and

CFCs must be used.

All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included. $)

Accessories for$(langAccessor WC cubicles:

KEMMLIT-door stopper hooks made of  aluminium 401$)/E

$$NNN

$(h1ii67.20$)			$(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-wall hooks made of aluminium 402$)/E

$$NNN

$(h1ii67.30$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-paper roll holders made of aluminium 408$)/E

$$NNN

$(h1ii67.40$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-replacement toilet roll holders made of aluminium 409$)/E

$$NNN

$(h1ii67.50$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............$)

$(langKEMMLIT-WC brush set made of aluminium 410
 $)/E$$NNN

$(h1ii67.60$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

Accessories are also available in stainless steel and nylon.

/E$$GRUEnd

----------------------

Cubicle system$(kurz9Cubic hardcell aluminium$)	$(gb.............

$)

============/E