68$)	$(kurz9Cubicle system hardcell stainless steel $)

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:

Asymmetric folded stainless steel sandwich elements,

42 mm thick, with a static connection and

vandal-resistant hardware. TÜV-approved (GS seal

of approval).

Options:
hardcell stainless steel 4.1: Anodised aluminium profiles

hardcell stainless steel 4.4: Stainless steel profiles

Height:
2000 mm, including 150 mm under panel clearance.
2135 mm, including 150 mm under panel clearance.

Other heights are also available.

Walls:
Stainless steel sandwich element, 42 mm thick according

to DIN EN 10 027-2. The stainless steel sheets are

canted on all 4 sides.

The inside of the elements is filled with a warp-resistant

inlay. Additional screw-proof inlays in the sandwich elements

- guarantee the long-lasting and solid fixing of accessories.

Profiles:
The horizontal stainless steel profile (frieze) does not run

- directly above the door front but rather is set back by

- 220mm.

- Alternatively profile (frieze) available in aluminium.

Wall connections are accomplished by shadow gaps.

Supporting legs:
The supporting legs are set back 220 mm.

Made entirely of stainless steel with a diameter of  20 mm.

Adjustable in height. Force-fit connection to the partition with M12

Screw fitting. The durable stainless steel cover sleeve can easily be

lifted for cleaning purposes.

Also available in corrosion-free aluminium, satin anodised

with stainless steel core and plate. Durable cover

sleeve made of anodised aluminium. Cover sleeve can easily be

lifted for cleaning purposes

Doors:
Stainless steel sandwich elements, 42 mm thick, DIN EN

10 027-2, rebated (same construction as walls).

- Hinges:

2 robust, 2–roller-rising-butt-edge hinges made

of aluminium,  (20 mm in diameter), for automatic

closing of the door, satin anodised.

Alternatively 2 solid 2-roller edge hinges (20mm

in diameter), made of satin anodised aluminium.

- Alternatively decorative 2-roller edge hinges made of

- stainless steel (12 mm in diameter).

- Furniture/hardware:

Single latch lock WC. Door knobs, red/white

indicator bolt with emergency release mechanism

made of satin anodised aluminium, in stainless

steel or impact and shatter-resistant and colour-coordinated

nylon.

Double latch lock WC, safety lever handles, 23 mm in diameter. Red/white

indicator bolt with emergency release mechanism

made of natural anodised aluminium, stainless steel

or impact and shatter resistant nylon.

Colours:
Colours of doors and walls according to the cell colour chart.

Environmental friendliness:
Only environmentally friendly materials without PVC

and CFCs must be used.

All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included.

68.10 			$)			$(menge0,000$) no.$(einhN$) 	$(d276$)$(ep.............$)	$(gb.............

Accessories for WC cubicles$(lang:

KEMMLIT-door stopper hooks made of  aluminium 401$)/E

$$NNN

$(h1ii68.20$)			$(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-wall hooks made of aluminium 402$)/E

$$NNN

$(h1ii68.30$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-paper roll holders made of aluminium 408$)/E

$$NNN

$(h1ii68.40$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-replacement toilet roll holders made of aluminium 409$)/E

$$NNN

$(h1ii68.50$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............$)

$(langKEMMLIT-WC brush set made of aluminium 410
 $)/E$$NNN

$(h1ii68.60$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

Accessories are also available in stainless steel and nylon.

/E$$GRUEnd

----------------------

$(kurz9 Cubicle System hardcell stainless steel $)	$(gb.............

$)

============/E