$(h1ii58$)64	$(kurz9 Cubicle System softcell aluminium$)

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:

Aluminium sandwich element with rounded

front components, polygonal cubicle shape, 42 mm

thick, corrosion-resistant with 3-component varnish coating

(Crystal Coat), with a static connection and

elegant hardware. TÜV-approved (GS-seal of approval).

The front of each cubicle can either be recessed or

brought forward from the adjacent cubicle by 100mm.

Options:
softcell Aluminium 3.1: Anodised aluminium profiles

softcell Aluminium 3.4: Stainless steel profiles

Height:
2000 mm, including 150 mm under panel clearance.
2135 mm, including 150 mm under panel clearance.

Other heights are also available.

Walls:
42 mm thick, corrosion-resistant aluminium-sandwich

element. Environmentally friendly 3-component varnish coating

with a non-sensitive structure (Crystal Coat), scratch-

and UV-resistant according to DIN 53384 A. The

aluminium alloy is seawater-resistant according to DIN 1725.

- The aluminium sheets are canted on all 4 sides.

The inside of the elements is filled with a warp-resistant inlay.

Additional screw-proof inlays guarantee the long-lasting  and

solid fixing of accessories.

Profiles: 
The horizontal aluminium profile (frieze) does not run directly

above the door front but is rather set back 270mm.

- Alternative stabilising profile (frieze) made of stainless steel.

The wall connections are accomplished by shadow gaps.

Supporting legs:
The supporting legs are set back 270mm,

made entirely of stainless steel with a diameter of 20mm.

Adjustable in height. Force fit connection to the partitions

with M12 screw fitting.  Durable stainless steel cover

sleeve can easily be lifted for cleaning purposes.

Alternatively made of corrosion-free aluminium, satin

Anodised, with stainless steel core and plate.

Durable anodised aluminium cover sleeve can easily

be lifted for cleaning purposes.

Doors:
Aluminium sandwich element, 42 mm thick,

corrosion-resistant, rebated (same construction as the

wall elements).

- Hinges:

2 robust, 2–roller-rising-butt-edge hinges made

of aluminium (20 mm in diameter) for automatic

closing of the door, satin anodised.

Alternatively 2 solid 2-roller edge hinges (20mm

in diameter). Made of aluminium satin anodised.

- Alternatively decorative 2-roller edge hinges made of

- stainless steel (12 mm in diameter).

- Furniture/hardware:

- Single latch lock WC, door knobs, red/white indicator bolt

- with internal emergency release mechanism in stainless

- steel, in aluminium satin anodised, or in impact and

- shatter-resistant and colour-coordinated nylon.

Double latch lock WC, safety lever handle (23 mm in diameter),

red/white indicator bolt with internal emergency release

mechanism in aluminium satin anodised, in stainless steel

or in impact and shatter-resistant and colour-coordinated

nylon.

Colours:
Colours of doors and walls according to the cell colour chart.

Environmental friendliness:
Only environmentally friendly materials must be used. No PVC

or CFCs.

All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included.

64.10 			$)			$(menge0,000$) no.$(einhno. 	$(d276$)$(ep.............$)	$(gb.............

Accessories for WC cubicles$(lang:

KEMMLIT-door stopper hooks made of  aluminium 401$)/E

$$NNN

$(h1ii64.20$)			$(menge0,000$) no.$(einhno.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-wall hooks made of aluminium 402$)/E

$$NNN

$(h1ii64.30$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-paper roll holders made of aluminium 408$)/E

$$NNN

$(h1ii64.40$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-replacement toilet roll holders made of aluminium 409$)/E

$$NNN

$(h1ii64.50$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............$)

$(langKEMMLIT-WC brush set made of aluminium 410
 $)/E$$NNN

$(h1ii64.60$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

Accessories are also available in stainless steel and nylon.

/E$$GRUEnd

----------------------

$(kurz9 Cubicle System softcell aluminium $)	$(gb.............

$)

============/E