63$)	$(kurz9 Cubicle System softcell steel$)

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:

Steel sandwich element with rounded front

components, polygonal cubicle shape, 42 mm thick,

hot galvanised, with a static connection

and elegant hardware. TÜV-approved (GS-seal of

approval).

The front of each cubicle can either be recessed or

brought forward from the adjacent cubicle by 100mm.

Options:
softcell steel 1.1: Anodised aluminium profiles

softcell steel 1.4: Stainless steel profiles

Height:
2000 mm, including 150 mm under panel clearance.
2135 mm, including 150 mm under panel clearance.

Other heights are also available.

Walls:
42 mm thick steel sandwich element, hot galvanised. 
S 250 GD + Z-275 according to DVV and DIN EN 10142,

DIN EN 10147 and DIN EN 10143.

Visible polyester varnish coating approx. 25 µm thick in

white, grey, metal grey and black. With additional

protective varnish coating on the inside 7-10 µm .

Alternatively visible polyester varnish coating in special crystal

effect approx. 50 m thick. UV-resistant due to embedded

acrylic glass parts. In colours silverstar, redstar, bluestar,

brownstar and greenstar.  With additional protective varnish-

coating on the inside 7-10 µm.

The steel sheets are canted on all 4 sides. The inside of the

elements is filled with a warp-resistant inlay. Additional screw-proof inlays

guarantee the long-lasting and solid fixing of accessories.

Profiles: 
The horizontal aluminium profile (frieze) does not run directly

above the door front but is rather set back 270mm.

- Alternative stabilising profile (frieze) made of stainless steel.

The wall connections are accomplished by shadow gaps.

Supporting legs:
The supporting legs are set back 270mm. Made entirely

of stainless steel with a diameter of 20mm.

Adjustable in height. Force fit connection to the partitions

with M12 screw fitting.  Durable stainless steel cover sleeve

can easily be lifted for cleaning purposes.

Alternatively made of corrosion-free aluminium, satin anodised

with stainless steel core and plate. Durable anodised aluminium

cover sleeve can easily be lifted for cleaning purposes.

Doors:
Steel sandwich element, 42 mm thick, hot galvanised, rebated

(same construction as the wall elements).

- Hinges:

2 robust, 2–roller-rising-butt-edge hinges made

of aluminium,  (20 mm in diameter), for automatic

closing of the door, colour satin anodised.

Alternatively 2 solid 2-roller edge hinges (20mm

in diameter). Made of aluminium satin anodised or

powder-coated.

- Alternatively decorative 2-roller edge hinges made of

- stainless steel (12 mm in diameter).

- Furniture/hardware:

- Single latch lock WC. Door knobs, red/white indicator bolt with

- internal emergency release mechanism in aluminium satin

- anodised, stainless steel, or in impact and shatter-resistant

and colour-coordinated nylon.

Double latch lock WC. Safety lever handle (23 mm in diameter).

Red/white indicator bolt with internal emergency release

mechanism in aluminium satin anodised, in stainless steel

or in impact and shatter-resistant and colour-coordinated

nylon.

Colours:
Colours of doors and walls according to the cell colour chart.

Environmental friendliness:
Only environmentally friendly materials must be used.

No PVC or CFCs.

All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included.

$)

Accessories for WC cubicles$(lang:

KEMMLIT-door stopper hooks made of  aluminium 401$)/E

$$NNN

$(h1ii63.20$)			$(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-wall hooks made of aluminium 402$)/E

$$NNN

$(h1ii63.30$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-paper roll holders made of aluminium 408$)/E

$$NNN

$(h1ii63.40$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-replacement toilet roll holders made of aluminium 409$)/E

$$NNN

$(h1ii63.50$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............$)

$(langKEMMLIT-WC brush set made of aluminium 410

$(h1ii63.60$)			                             $(menge0,000$) no.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............$)

Accessories are also available in stainless steel and nylon.

$)/E$$NNN

$(h1ii$)			                             $(menge0,000$) No.$(einhNo.$) $)	$(d276$)$(ep.............$)	$(gb.............

$)

============/E

Cubicle system softcell steel