5$(h1ii7$)	Cubicle system variocell steel

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:

Steel sandwich elements, 30 mm thick, hot-dip

galvanised with a static connection and

vandalism-resistant hardware.

TÜV-approved (GS seal of approval).

Options:
variocell steel 1.1: Anodised aluminium profiles
variocell steel 1.2: Powder-coated aluminium profiles

variocell steel 1.4: Stainless steel stabilisator headrail

Height:
2000 mm, including 150 mm under panel clearance.

2135 mm, including 150 mm under panel clearance.

Other heights are also available.

- Suspended cubicle construction without supporting legs

- as an easy-to-clean alternative.

Support-optimised cubicle construction

with supporting legs set back in the partition.

Floor-to-ceiling construction for a maximum privacy.

Walls:

Steel sandwich elements, 30 mm thick hot-dip galvanised

according to S 250 GD + Z-275 according to  DVV,

DIN EN 10142 resp. DIN EN 10147 and DIN EN 10143.

Visible polyester varnish-coating approx. 25 µm
thick in white, grey, metal grey and black.

With additional protective coating on the inside 7-10 µm.

Alternatively visible polyester varnish-coating in special crystal

effect approx. 50 m thick. UV-resistant due to embedded

acrylic glass parts. In colours silverstar, redstar, bluestar, brownstar

and greenstar.

With additional protective coating on the inside 7-10 µm .

- The steel sheets are canted  on all 4 sides. The inside of the

- elements is filled with a warp-resistant inlay.

- Screw-proof inlays in the sandwich elements guarantee the long-lasting  and solid

fixing of accessories.

Alternatively for better fire prevention, the inside of the

elements can be filled with inflammable metal inlays.

Profiles:
All profiles are made of aluminium, rounded (2mm radius)

to reduce the risk of injury. Warp-resistant, elegantly rounded

headrail (55/43mm)	runs directly above the door front.

Wall-connection profiles made from u-channels to allow for

adjustments site.

The headrail is also available as a smooth-surfaced

rectangular shape in aluminium (42/43mm).

The headrail is also available as a rectangular

shape in stainless steel (40/30 mm).

- The headrail can optionally be set back

- by 130mm rather than running directly above the

- door fronts.

Supporting legs:

20 mm in diameter, made entirely of stainless steel,

adjustable in height. Force-fit connection to the

partition by means of M12 screws. The durable stainless

steel cover sleeve can easily be lifted for cleaning purposes.

Also available in corrosion-resistant aluminium.

Satin anodised or power-coated with stainless

steel core and plate. Durable cover

sleeve made of anodised aluminium.

Cover sleeve can easily be lifted for cleaning purposes.

As a further option, made of corrosion-resistant

aluminium, powder-coated with stainless steel core

and plate. Durable cover sleeve made of colour-coordinated

nylon. The cover sleeve can easily be lifted for cleaning purposes.

Doors:
Steel sandwich elements, 30 mm thick, hot-dip

galvanised (same construction as walls) with inner, vertical

aluminium profiles. The doorstop profiles are made of aluminium

with integrated sound absorption rubber stop

to reduce closing noise.

Inward-opening doors can optionally be fitted with a

safety finger-protection profile on the side of the

hinges (with pivot hinges instead of standard hinges,

maximum opening angle = 100°).

Hinges:

2 robust, 2–roller-rising-butt-edge hinges made

of aluminium,  20 mm in diameter, for automatic

closing of the door, colour satin anodised

- or power coated.

Alternatively 2 solid 2-roller edge hinges- (20 mm in diameter),

made of aluminium satin anodised or powder-coated.

- Alternatively decorative 2-roller edge hinges made of

- stainless steel (12 mm in diameter).

- Furniture/hardware:

Single latch lock WC. Door knobs, red/white

indicator bolt with emergency release mechanism

made of satin anodised aluminium, in stainless

steel or impact- and shatter-resistant, colour-coordinated nylon.

As an option, double latch lock WC. Safety

lever handle, 23 mm in diameter. Red/white

indicator bolt with emergency release mechanism

made of natural anodised aluminium, stainless steel

or impact and shatter-resistant, colour-coordinated nylon.

Single turning knob made of satin anodised aluminium

with red/white indicator.

Colours:
Wall and door elements, profiles, hardware, supporting legs and accessories

in the same consistent colours according to the cell colour range.

Environmental friendliness:
Only environmentally friendly materials without PVC and

CFCs are used.

All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included.

$)

57.10 			$)			$(menge0,000$) no.$(einhno. 	$(d276$)$(ep.............$)	$(gb.............

- Further optional fittings at a surcharge:

- Sliding door

57.12 			$)			$(menge0,000$)  no.$(einhno. 	$(d276$)$(ep.............$)	$(gb.............

- Space-saving folding door

57.13 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

- Upper panel made of steel sheet

57.14 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

- Glass upper panels

57.15 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

- Swinging door (saloon door)

57.16 			$)			$(menge0,000$) $(einh no.	$(d276$)$(ep.............$)	$(gb.............

$(h1ii

$(lang Accessories for WC cubicles$(langAcec:

KEMMLIT door stopper hooks made of nylon 301$)/E

$$NNN

$(h1ii57.20$)			 $(menge0,000$) $(einh no. $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-Wall hooks made of nylon 302$)/E

$$NNN

$(h1ii57.30$)			                             $(menge0,000$) $(einh no. $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT toilet roll holder made of nylon 308$)/E

$$NNN

$(h1ii57.40$)			                             $(menge0,000$) $(einh no. $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT replacement toilet roll holder made of nylon 309$)/E

$$NNN

$(h1ii57.50$)			                             $(menge0,000$) $(einh no. $)	$(d276$)$(ep.............$)	$(gb.............$)

$(langKEMMLIT-WC-toilet brush set made of nylon 310
 $)/E$$NNN

$(h1ii57.60$)			                             $(menge0,000$) $(einh no. $)	$(d276$)$(ep.............$)	$(gb.............

Accessories are also available in aluminium or stainless steel.

Accessories for changing room cubicles:

KEMMLIT-bench seating for changing room

cubicles made of HPL solid grade laminate. $)

$$NNN

$(h1ii57.70$)			                             $(menge0,000$) $(einh no. $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT-Under bench central locking system

or overhead central locking system or for the

simultaneous locking of both cubicles

with emergency release and clearly visible

vacant/occupied indicator (red/white)

$)

$$NNN

$(h1ii57.80$)			                             $(menge0,000$) $(einh no. $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT door stopper hooks for

changing cubicles made of nylon no. 301

57.90$)			                             $(menge0,000$) $(einh no. $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT crystal mirror for changing cubicles

with sealed edges 8 mm thick. Corners rounded 300 x 400 mm.

Gluing is prohibited.

Theft proof screw connection. $)/E

$$NNN

$(h1ii57.100$)			                             $(menge0,000$) $(einh no. $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT shelf for changing cubicles

made of nylon no. 312,

Dimensions  300 x 50 mm.

Theft proof screw connection. $)/E

$$NNN

$(h1ii57.110$)			                             $(menge0,000$) $(einh no. $)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT anti-climb protection for changing cubicles. $)

$(h1ii57.120$)			                             $(menge0,000$) $(einh no. $)	$(d276$)$(ep.............$)	$(gb.............$)

/E$$GRUEnd

----------------------

Cubicle system$(kurz9Cubicle  variocell steel$)	$(gb.............

$)

============/E