13$(h1ii$)	$(kurz9Cubicle system NOXX smart

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:

Panels and doors made of 10mm thick toughened

safety glass with full-surface silk-screen printing.

Edges polished.

The construction must be TÜV-tested to ensure a high

level of stability, the test certificate must be submitted

together with the quotation. As proof of safety an

additional GS (tested safety) test must be provided.

Furniture and profiles made of satin anodised aluminium

(E6/EV1). Easy to clean and absolutely corrosion-free

(even surface rust).
Softclose-system – for a pleasant quiet closing of the doors.

The cubicle front is smooth surfaced with precise joints.

The wall connection of the front panels is accomplished

with glass holders.

The square horizontal aluminium profile is situated

approximately 107 mm behind the cubicle front.

The supporting legs are recessed in the partition panel

and positioned flexibly (to be able to respond to tile patterns

or underfloor heating):

Design option: recessed in the partition panel and therefore

not visible from the outside.

Optional as a special variant: supporting legs in the front with

lower corner edge protection for particular heavy use.

The supporting structure consists of a warp resistant

Connection system.

Partition and side panels are furnished with solid

aluminium partition holders.

The front panels are furnished with solid aluminium

clamping holders on the upper edge

All screw connection are done with special glass screws.

Height:
2135 mm (upper edge glass), incl. 100mm under panel clearance.

Clear passage height 2163mm.

Other heights are available.

Supporting legs:
Screwed to floor supporting legs (Ø 20 mm) invisibly and steplessly

adjustable in height (+25mm/-5mm, to be able to respond to declining

floors), made of satin anodised aluminium.

Stainless steel core and stainless steel floor plate as well as

solid cover sleeve made of aluminium.

The connection to the partition holders is accomplished with

M12 threaded bolts for maximum stability.

As a special option the supporting legs can be glued to the floor in the

front panel (avoiding to drill through the floor sealing).

Grouted supporting legs are not allowed.

Doors:
Doors are self closing, with integrated Softclose-system

for a pleasant quiet closing of the door.

Furniture:

Sturdy, impossible to unhinge Softclose-3-roller-hinges

with individually adjustable closing force made from aluminium

(for a pleasant quiet closing oft he door),

satin anodised (E6/EV1).

Hinge fastening with special glass screws.

The hinges have been tested and ensure a minimum of

200.000 openings of the doors, a certificate for this must be

provided.

Single hand turning knob with WC-lock on the inside,

indicator red/white with emergency release on the outside.

Environmental acceptability:
Only environmentally friendly material may be used

without PVC and CFCs.
All used materials are 100% recyclable.

The colours are lead and cadmium free.

Colours:
Selection of 12 colours according to the NOXX smart -
colour concept.

Further RAL colours are available after prior sample approval.
Profiles, hinges, furniture 
made of satin anodised aluminium.

Cubicle system consisting of:

Front panel		............ mm 	.......... Nos.

Doors incl. furniture			.......... Nos.

Partition panels		............ mm 	.......... Nos.

$(einhNos. of units	$(menge0,000$) $(einhNos.$)	$(d276$)$(ep............. 	            .............$)

Prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included.

Accessories:

/E$$NNN

$(h1ii13.20	Enamelled glass urinal screen wall hung	$(menge0,000$) $(einhnos$)	$(d276$)$(ep............. 	......

made of 10 mm TSG-glass available in all

12 NOXX smart colours with 2 aluminium

panel holder,

Size: (height/width) 1475 mm/435 mm,

- 650 mm under panel clearance.

$(h1ii13.30$)	Enamelled glass urinal screen with aluminium	$(menge0,000$) $(einhnos$)	$(d276$)$(ep............. 	.............$)$(gb$)$(langEdelstahl-

supporting legs made of 10 mm TSG-glass available

in all 12 NOXX smart colours with 2 aluminium
panel holder,

Size: (height/width) 1475 mm/ 435 mm,

- 100 mm under panel clearance.

$$NNN

$(h1ii13.40 	KEMMLIT-door stopper hook 	0,000$) $(einhnos$)	$(d276$)$(ep............. 	.............$)	$(gb$)

made of aluminium

(fitted to the glass with special glue)	
$(langEdelstahl-$)		 $)	$(menge

/E$$NNN

$(h1ii13.50 	$)KEMMLIT-wall hook small	0,000$) $(einhnos$)	$(d276$)$(ep.............$)	$(gb.............

made of aluminium			$(menge$)

$(lang(fitted to the glass with special glue)$)

/E$$NNN

$(h1ii$)$$NNN$(h1ii13.60$)		KEMMLIT-paper roll holder				0,000$) $(einhnos$)    $(d276$)$(ep.............$)		           $(gb.............

made from aluminium		$(menge$)

(fitted to the glass with special glue)$(lang$)

/E

$$NNN

$(h1ii13.70$)		KEMMLIT-Replacement paper roll holder 	$(menge0,000$) $(einhnos$)	$(d276$)$(ep.............$)	$(gb.............$)

made of al$(langmade of aluminium

(fitted to the glass with special glue)
/E

$$NNN

$(h1ii13.80$)		KEMMLIT-brush set with brush holder	$(menge0,000$) $(einhnos$)$(d276$)	$(ep.............$)	$(gb.............$)

made of a$(langmade of aluminium

$(h1ii13.90$)		Hygiene-waste bin with 		0,000$) $(einhnos$)$(d276$)	$(ep.............$)	$(gb.............
dispenser compartment for hygiene bags		$(menge$)

Bürstenhalter 511

----------------------

$(kurz9Cubicle system NOXX smart$)	$(gb.............$)

============

/E