13$(h1ii$)	Cubicle system$(kurz9Cub NOXX GL

$(lang

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Option in glass:
NOXX GL  – Suspended cubicle – 
TÜV-approved (GS-seal of approval).

Height:
Front: 2135 mm incl. 80 mm under panel clearance.
(Other heights are not available)

Construction:

Cubicle system with few supporting legs creating a levitating effect.

The head profile cannot be seen from the outside of the cubicle.

TÜV-approved (GS-seal of approval).

The cubicle front is smooth surfaced with precise joint pattern.

Connection of the front panel to the walls by means of

glass holders.

The horizontal aluminium profile (frieze) does not run directly

above the door fronts, but is set back by 70mm.

The supporting legs are set back by 130mm.

Supporting structure by means of T-connection system.

Partition and side panels are fitted with solid aluminium

panel holders (length: 170mm). The upper and lower edges

of the pilasters (front panels) are fitted with solid aluminium

connecting angles and terminal strip. The partition and side

panels are invisibly connected to the pilaster (front panel) with

a force-fit connection by means of  the T-connection system.

All screws are special glass screws.

Partitions and pilasters (front panels):
Toughened safety glass (ESG) 10 mm.

Partition and side panels are also available in HPL 10mm

or in a combination of glass and HPL.

Supporting legs:

Supporting legs screwed to the floor (20 mm in diameter)

adjustable in height, made of brushed stainless steel with

stainless steel foot plate and durable stainless steel cover

sleeve.

The supporting legs are fixed by means of M12 threaded bolts

in the partition wall holders.

(Supporting legs mortared into the floor are not available)

Doors:
Toughened safety glass (ESG) 10 mm.

Doors sit flush with front with integrated rubber sound

absorption profile on the side of the door lock and with

opaque joints.

Furniture/hardware:
 Solid 3-roller edged hinges made of aluminium, satin

anodised, colour E6/EV1. Fastening of the hinges by

means of special screws for glass.

Single handed turning door knob with WC-door lock on

the inside, red/white indicator including safety opening.

Free/occupied indicator designed as „X“.

Environmental friendliness:
Only environmentally friendly materials without PVC and

CFCs are used.

Colours:
Choice of 11 colours according to the NOXX colour charts.

Profiles, hinges and hardware in aluminium satin anodised.

Cubicle system  consisting of:

Pilaster (front panel):		            ............ mm 	.......... no.

Doors with hardware:			                          .......... no.

Partitions:	                                      ............ mm 	.......... no.

Number of sets:$(einhQuantit	 ............ $(menge$(einhno. $)	$(d276$)$(ep

All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included.

Accessories:

/E$$NNN

$(h1ii13.10$)	Toughened glass urinal screen wall-hung	0,000$) $(einh no.$)	$(d276$)$(ep............. 	 .............$(menge$(gb$)

made of$(langEdelstahl-mad 10 mm ESG glass in all 11 NOXX-colours$)	$(d276$)$(ep

with  2 aluminium panel holders

Dimensions: (height/width) 1475 mm/435 mm,

- 650 mm under panel clearance.

$(h1ii13.20$)	Toughened glass urinal screen with stainless steel

supporting leg. Made of 10 mm ESG glass in all	$(menge0,000$) $(einh no.$)	$(d276$)$(ep............. 	.............$)$(gb$)$(langEdelstahl-

11 NOXX-colours with  2 aluminium panel holders.

Dimensions: (Height/width) 1475 mm/ 435 mm,

- 80 mm under panel clearance.

$$NNN

$(h1ii13.30 	KEMMLIT door stopper hooks	0,000$) $(einh no.$)	$(d276$)$(ep............. 	.............$)	$(gb$)

made of stainless steel $(langEdelstahl-$)		made of stain 501.$)	$(menge

/E$$NNN

$(h1ii13.40 	$)KEMMLIT wall hooks made of	$(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)	$(gb.............$)

$(lang stainless steel $(langEdelstahl-$)		made of stain 503.$)

/E

$$NNN

$(h1ii13.50$)	KEMMLIT ashtray made of stainless steel $(langEdelstahl-$)		made of stain 507.	$(menge0,000$) no.$(einhNo$)	$(d276$)$(ep.............$)	$(gb.............$)

$(lang$)

/E

$$NNN

$(h1ii13.60$)	KEMMLIT toilet roll holder made of stainless	$(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)	$(gb.............$)

steel $(langEdelstahl-$)		made of stain 508.$(lang$)

/E

$$NNN

$(h1ii13.70$)		KEMMLIT replacement toilet roll holder made	$(menge0,000$) $(einh no.$)	$(d276$)$(ep.............$)	$(gb.............$)

$(lang of stainless steel 509.

/E

$$NNN

$(h1ii13.80$)		KEMMLIT toilet brush set made of stainless	$(menge0,000$) $(einh no.$)$(d276$)	$(ep.............$)	$(gb.............$)

$(langsteel $(langEdelstahl-$)		made of stain 511.

Bürstenhalter 511

/E$$GRUEnd

----------------------

$(kurz9 Cubicle system$(kurz9Cub NOXX GL

$)	$(gb.............$)

============

/E