8$(h1ii$)	$(kurz9Cubicle system D-Bambino

$(langKEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:
Toilet cubicles for nursery and infant schools.

Accident safety guaranteed according to the

safety guidelines of the GUV. TÜV-approved.

(GS seal of approval). 

Height:
1400 mm incl. 120 mm under panels clearance

(standard).

Lower heights are available.

Partitions and pilasters (front panels):

30 mm thick solid melamine-coated particle board.
All edges are protected against impact with ABS

lipping 3mm thick (radius=2 mm). All corners are

rounded with a radius of 33 mm to reduce risk of

injury. Wall connection with powder-coated solid

aluminium panel holders, to allow minor

adjustments for tolerances on site.

Supporting Legs:
Height adjustable aluminium supporting legs

(20 mm in diameter) powder-coated with stainless

steel foot plate an durable nylon cover sleeve.

(Supporting legs glued to the floor are not allowed

for safety reasons).

Doors:

Same construction as partitions and pilasters (front

panels), with safety finger protection on both sides.

A 20mm wide soft finger protection profile on the lock

side and on the hinge side a continuous aluminium finger

protection profile. Swing-doors are not allowed, door

opening outwards only.

- The door fronts are available  in 3 different constructions

- according to the KEMMLIT brochures.

Hardware:

3-roller safety spring-loaded edge hinge, 20 mm in diameter,

with aluminium finger protection profile running between the

two hinges.

Door handles:

Safety ring handle made of impact- and crack-resistant

nylon with integrated rubber buffer as door stop.

Colours:

Colours of pilasters (front panels), partitions and

doors according to the KEMMLIT colour charts

(at least 15 colours)

- Profiles, hinges, door handles, supports and

accessories are available in the same colours

according to the KEMMLIT colour charts (at

least 6 colours).

- All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included.

- Accessories:

/E$$NNN

$(h1ii			$)	$(menge $)                    KEMMLIT paper roll holder made of nylon 308$)

$(d276$)

8.10                                                                                        0,000 no.             .............$)	                           $(gb.............$(ep$)

$(lang

KEMMLIT replacement paper roll holder made of nylon 309

/E

8.20$$NNN

$(h1ii$)	                       $(menge0,000$) no.$(einh$)	$(d276$)$(ep.............$)	  $(gb.............$)

$(lang$)

/E$$NNN

$(h1ii                    $(menge $)	$(langKEMMLIT toilet brush set, angular, made of nylon 310$)

$(d276$)

8.30		   0,000 no               ..............$)	                           $(gb.............$(ep$)

/E

$$GRUEnd

----------------------

Cubicle System D-Bambino$)	$(gb.............$)

============

/E