70$(h1ii$)	$(kurz9Cubicle System B-Bambino Exklusiv

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:
Cubicle system for daycare centres, nursery schools,

kindergarten and primary schools.

Designed with an educational idea in mind.

Accident safety guaranteed according to the

safety guidelines of the GUV. TÜV-approved. 

Height:
1400 mm incl. 80 mm or 120 mm under panel clearance

Lower heights are possible upon request.

Front panels (pilasters) and partition panels:
13 mm solid HPL-panels according to DIN EN 438

(DIN 16926) with a white core to easily spot dirt and soiling,

for ease of cleaning for an overall better hygiene.

Alternative option: Solid HPL panels with a black core, they are

water-resistant, rot-proof, vandalism-proof and easy to care for.

Corners are rounded (2 mm).

Wall connections via solid aluminium brackets (55+55+8 mm).

Attachment with 4 stainless steel screws for additional stability and

for injury prevention.

Supporting legs:
Legs are Ø 20 mm, infinitely adjustable, made of anodised aluminium and come  
with stainless steel foot plates and stainless steel core threading to be firmly screwed

onto cubicle elements using a  M12 thread, torsion resistant locked in place.

Plastic connectors are not allowed due to the exposure to cleaning agents.

Covering rosettes are made of anodized aluminium.

Doors:
Same construction as wall elements. A 20 mm finger protection profile
on the hinge side of the door and a continuous aluminium finger protection

element on the swing side of the door prevents any gaps and thus injuries.

Doors can be opened outwards 180°.

There are 3 different themes available. (see Kemmlit Bambini Brochure)

Hinges:

Non-detachable 3-roller safety spring-loaded edge hinge, 20 mm in diameter,

attached with 4 square stainless steel screws. Hinges come with a

stainless steel centre and a special sleeve for additional finger protection as well as a

continuous aluminium finger protection element between the hinges.

Solid anodised aluminium attachement brackets  (55+55+8 mm), which are attached with four square stainless steel screws. Optionally round screws are available.

Door knobs:

Angled Safety-ring-handle with integrated rubber buffer as door stop.

Door stops on the door panel are not allowed for safety reasons.

Optional: 26 different types of door handles matching 3 different design themes.

Colours:
Walls and door panels are also available in a white surface with a white core

Optional:  KEMMLIT-surface design according to the KEMMLIT Bambino design themes

/ KEMMLIT- colour chart.

- Profiles, hinges, door handles, supports and

accessories according to the KEMMLIT colour charts (at

least 6 colours).

- All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included.

$)

/E

$$NNN

$(h1ii70.10$)	$(menge0,000$) $(einhunits$)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langAccessories:

KEMMLIT- toiletpaper holder $)/E

$$NNN

$(h1ii70.20             made of nylon 308$)	$(menge0,000$) units$(einh$)	$(d276$)$(ep.............$)	$(gb.............

$)

$(langKEMMLIT- spare roll holder $)/E

$$NNN

$(h1ii70.30             made of nylon 309$)	$(menge0,000$) $(einhunits$)	$(d276$)	$(ep.............$)	$(gb.............

$)

$(langKEMMLIT- toilet brush holder square$)

70.40             made of nylon 310$)	$(menge0,000$) $(einhunits$)	$(d276$)$(ep.............$)	$(gb............./E

$$GRUEnd

----------------------

$(kurz9CuDCccCCubicle System B-Bambino Exklusiv$)	$(gb.............$)

============

/E