10$(h1ii$)	Cubicle system $(kurz9 cubicle system B-Bambino

$(langmade byKEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Construction:
Toilet cubicles for nursery and infant schools.
Accident safety guaranteed according to the safety

guidelines of the GUV. TÜV-approved.

(GS seal of approval).

Height:
1400 mm incl. 120 mm under panel clearance.

Lower heights are available.

Partitions and pilasters (front panels):
13 mm thick solid grade laminate (HPL) panels according

to DIN EN 438 (DIN 16926). Water-resistant, decay-proof,

resistant to damage through misuse and very easy to

maintain. All corners are rounded, with a radius of 30 mm to reduce risk of injury.

Wall connection with powder-coated solid aluminium panel

holders, to allow minor adjustments for tolerances on site.

Supporting legs:
Height adjustable aluminium supporting legs (20 mm in

diameter), powder-coated, with stainless steel plate and

durable nylon cover sleeve. (Supporting legs glued to the

floor are not allowed for safety reasons.)

Doors:

Same construction as partitions and pilasters (front

panels), with safety finger protection on both sides.

A 20 mm wide finger protection profile on the lock side

and a continuous aluminium finger protection profile on the

hinge side. Swing-doors are not allowed, door

opening outwards only.

The door fronts are available in 3 different constructions

(according to the KEMMLIT brochure).

Hardware:

3-roller spring loaded edge hinge, 20 mm in diameter,

with continuous aluminium finger protection

profile between the hinges.

Door handles:

Safety ring handle made of impact- and

crack-resistant nylon with integrated rubber

buffer as door stop.

Colours:

Colours of doors, pilasters (front panels) and

partitions according to the KEMMLIT colour

charts (at least 15 colours).

Profiles, hardware, supporting legs and

accessories available in the same continuous

colours according to the KEMMLIT colour chart

(at least 6 colours).

All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included.

Accessories:

/E

KEMMLIT paper roll holder made of nylon 308$)

10.10

$$NNN

$(h1ii$)	                   0,000 no.$(menge                   .............$)	$(gb.............$)	$(d276$)$(ep$(lang

$(langKEMMLIT replacement paper roll holder made of nylon 309$)

10.20/E

$$NNN

$(h1ii$)	                   $(menge0,000$) $(einh no.$)	$(d276$) $(ep.............$)	$(gb.............$)

$(langKEMMLIT toilet brush set, angular, made of nylon 310

10.30

0,000$) $(einh no.$)	$(d276$) $(ep.............$)	$(gb.............$)

$$GRUEnd

----------------------

Cubicle system B-Bambino$)	$(gb.............$)

============

/E

/E