10$(h1ii$)	Cubicle$(kurz9 System Type D Bambino,

Adult Cubicles

KEMMLIT UK
Bays Platt
Skirmett

Henley on Thames

Oxfordshire

RG9 6TD
Telephone: 01491 638606
info@kemmlituk.com - www.kemmlituk.com

Design:

Kindergarten WC cubicle system. Accident safety guaranteed according to the safety guidelines of the GUV. TÜV-approved (GS seal of approval).

Height:

1970 mm including 120 mm ground clearance.

Choice of 1400 mm door height incl. 120 mm under panel clearance.

Walls:

30 mm thick melamine resin laminated full chipboard panels.

All visible edges are protected with 3 mm thick shock-proof ABS bandings (radius = 2 mm), impact-proof protection

The lower corners are rounded off, 33 mm radius, in the passageway area in order to avoid accidents.

Edging:

Aluminium edging (2 mm radius), rounded off

and accident proof. The stabiliser at 55/43 mm

over the door front is elegantly rounded off.

Attaches to wall with U-shaped rail

to balance constructional tolerance.

Feet:

Ø 20 mm, height adjustable, aluminium

powder-coated with stainless steel plate and weight-bearing

cover-plate made of nylon.

Doors:

In the same construction as the walls, dampened door closing and corners rounded off  to avoid accidents, 3 mm radius, 3 door fitting variants are possible.

Door hinges: A choice of 3 variants

1)	2 sturdy 2-roll-bands made of aluminium, Ø 20 mm 2-bolt WC lock. Safety latch, 23 mm diameter. Red/white indicator with emergency bolt opening, made of impact- and break-proof nylon.

2)	3-roll safety spring hinge, Ø 20 mm, with

aluminium tubing straight through between the

bands to avoid trapped fingers.

2-bolt WC lock. Safety latch, 23 mm

diameter. Red/white indicator with emergency

bolt opening, made of impact- and break-proof

nylon.

3) 	3-roll safety spring hinge, Ø 20 mm, with aluminium tubing right through between the bands to avoid trapped fingers. Safety anti-finger-trap protection with soft lip profile on the lock side, with safety ring made of impact- and break-proof nylon with an integrated rubber dampener as the door hinge. Therefore, door only opens outwards, with closing lever lock as necessary, colour: white. For option 2, there is also the possibility of a door height suitable for children at 1400 mm including 120 mm under panel clearance.

Colours:

Wall and door colours in line with KEMMLIT colour chart (at least 15 colours)

Profiles, hinges, door handles, feet and accessories in the same universal colours

corresponding to the KEMMLIT colour concept

(at least 6 colours)

Prices including transportation and installation costs.

7.10		0,000 units

Accessories for WC cubicles:

KEMMLIT Door stop hook made from nylon 301

7.20		0,000 units

KEMMLIT Wall hook made from nylon 302

7.30		0,000 units

KEMMLIT Toilet roll holder made from nylon 308

7.40		0,000 units

KEMMLIT Replacement toilet roll holder made of nylon 309

7.50		0,000 units

KEMMLIT Toilet brush set, corner-shaped, made of nylon 310

7.60		0,000 units

Accessories also available in stainless steel.

/E$$GRUEnd

----------------------

$(kurz9	Cubicle System Type D Bambino,

Adult Cubicles$)	$(gb.............$)

============

/E